Bluetooth communication protocol V1.5

1. Summary

This protocol is used for bluetooth keycard lock, bluetooth pincode lock  and bluetooth keycard cabinet lock.

2. BLE data package（The maximum packet length is 20 bytes）

Frame format:

Frame header(1 byte) + length（1 byte）+ Command（1 byte）+ CmdData（N bytes) + Checksum code(2 bytes)

Frame header：Request header is 0x3a. Response header is 0x4b. The last package of request header should be 0x3b（When the device get 0x3b, it will disconnect the ble connection  immediately）.

Length：the byte count between command and checksum code.

Checksum：

There are two kind of checksum.

- Simple checksum. The simple checksum is used when request with the lock in setup mode, or used when select the lock before further command frame, or used when the lock response back to the host device.

Definition:

The ISO14443 CRC value in big endian from frame header to others, and with some salt bytes.

E.g.

Checksum = Function_ISO4443a_CRC( Frame header + length + ..... N bytes + Fix app ID)

Notice: the Fix app ID is 0x01. The salt bytes should not be include in the really send out bytes, it just used to affect the checksum.

- Sign checksum. The sign checksum is used in request with the lock in normal mode.

The ISO14443 CRC value in big endian from frame header to others, and with some salt bytes.

- E.g.

- Checksum = Function_ISO4443a_CRC( Frame header + length + ..... N bytes + 4 Bytes System ID + Fix app ID)

Notice: the Fix app ID is 0x01. The salt bytes should not be include in the really send out bytes, it just used to affect the checksum.

When the checksum wrong, the door lock will not response with anything.

;

Response error code:

0x00 Successful

0x01 Wrong address

0x02 Lock is not in setup mode

0x03 Invalid parameters

0x04 EKey expiry

0x05 EKey substituted

Random code:

The random code is used to keep every frame has a random frame id, and the lock will not response the request frame which has a invalid random code.After you send the select lock frame, the lock will generate a new random and response back to the host device.Then the host device should put this random code into the next request frame for validating purpose.

Idle connection:

The lock will force disconnect the bluetooth connection in 6 seconds after receiving the last valid frame data.

Time Val format（in big endian）: 32bit

Year：7bit  (current year - 2000)

Month：4bit (1~12)

Day of month：5bit (1 ~ 31)

Hour：5bit (0~23)

Minute：6bit (0~59)

Select lock frame

This frame should be sent out as the first frame after bluetooth connected. And if you need to keep the connection not idle, please issue this frame in every 5 seconds.

When the lock in setup mode, you have to use 0xFF for building id, floor id and lock id.

When the lock in normal mode, you have to use a right building id, floor  id and lock id for lock address validate purpose.

Short select request (Length: 0x06)：

0x3a + 0x6 + 0x01 + Building id（1Byte）+ Floor id（2Bytes）+ Lock id（2Bytes）+ Checksum（2Bytes）

Response:

0x4b + 0x04 + 0x01 + Next Random code（2Bytes）+ error code（1Byte）+ Checksum（2Bytes）

Long select request(Length: 0x7)：

0x3A + 0x7+ 0x01 + 0x01 + Building id（1Byte）+ Floor id（2Bytes）+ Lock id（2Bytes）+ Reserved options(1Byte) + Checksum（2Bytes）

Response(the lock will response as the follow definition when selecting successful. The short select response will send when selecting failure)：

0x4b + 0x0c + 0x01 + Next Random code（2Bytes）+ error code（1Byte）+ Firmware version(2Bytes） + Voltage(2Bytes, E.g.6.25V will present as :0x02+0x71) + clock in TimeVal format（4Bytes）+ Checksum（2Bytes）

Lock address setup frame（The lock only response this request when it work in setup mode）

Request：

0x3A + 0x0d+ 0x02 + Random code（2Bytes）+ System ID（4Bytes）+ Building ID（1Byte）+ Floor ID（2Bytes）+ Lock ID（2Bytes）+ Reserved Options（1Byte）+ Checksum（2Bytes）

Response：

0x4b + 0x04 + 0x02 + New random code（2Bytes）+ error code（1Byte）+ Checksum（2Bytes）

Setup time frame

This frame is used to setup the clock of lock.

Request：

0x3A + 0xa + 0x04 + Random code（2Bytes）+ Time Info（7Bytes）+ Checksum（2字节）

Time Info: Year(1Byte)+Month(1Byte)+Day Of month(1Byte) + Day of week(1Byte)+Hour(1Byte)+Minute(1Byte)+Second(1Byte)

Note: The year field should fill with current year - 2000.

Response：

0x4b + 0x04 + 0x04 + Next random code（2Bytes）+ error code（1Byte）+ Checksum（2Byte）

Setup ble name frame（The lock only response this request when it work in setup mode）

This frame is used to rename the ble name for lock. The ble name should be unique for ble lock searching purpose.Notice:the ble name should less than 16 characters in ASCII.the tail characters should be fill with 0.

Request:

0x3A + 0x13+ 0x03 + Random（2Bytes）+ Ble name（16Bytes）+ Checksum（2Bytes）

Response：

0x4b + 0x04 + 0x03 + new random code（2Bytes）+ error code（1Byte）+ Checksum（2Bytes）

Reset frame

This frame is used to reset the lock setting back to factory default.

Request:

0x3A + 0x4+ 0x05 + Random code（2Bytes）+ Clear options（1Byte）+ Checksum（2Bytes）

Clear options：Set as 0x01 means to reset the substitute code only. Others means to reset all.

Response：

0x4b + 0x04 + 0x05 + New random code（2Bytes）+ error code（1Byte）+ Checksum（2Bytes）

Unlock frame

This frame is used to unlock the lock.

Request:

0x3A + 0x10+ 0x06 + Random code（2Bytes）+ Unlock options（1Byte）+ reserved bytes（4Bytes）+ end date time（4Bytes）+ Begin date time（4Bytes）+ Checksum（2Bytes）

Unlock options：bit7 as 0x1 means to unlock and keep the door unlocks. As 0x0 means to unlock and lock again after 5 seconds；bit6 as 0x0 means to unlock deadbolt；bit5 as 0x1 means to enable the substitute flag（working as the substitute feature of key card）；bit4 as 0x1 means not to check with the end date time.

Response：

0xbb + 0x08 + 0x06 + New random code（2Bytes）+ error code（1Byte）+ First unlock event offset (2Bytes/start from zero）+ Unlock event count(2Bytes) + Checksum（2Byte）

Read the unlock event

Read frame

This frame is used to read the unlock event from the lock.

Request：

0x3A + 0x07+ 0x07 + Random code（2Bytes）+ Begin offset（2Bytes/start from zero）+ count（2Bytes）+ Checksum（2Bytes）

Normally we use 0xFFFF as the begin offset. And the lock will response the events which less than max count.

Negative response：

0x4b + 0x04 + 0x07 + Random code（2Bytes）+ error code（1Byte）+ Checksum（2Byte）

8 bytes positive response:

0x4b + 0x0D + 0x07 + new random code（2Bytes）+ event offset(2Bytes/start from 0，0xFFFF means the current response is the end of response. Note:the data contain in 0xFFFF frame is means nothing) +event data（8Bytes）+ Checksum（2Bytes）

16 bytes positive response:

Due to the pin code event data more than 8 bytes, so we uses two 8 bytes positive response frame to compose as one frame.for the events of key card, key, mobile unlock, we still uses a single 8 bytes positive response frame.

First 8 bytes positive response:

0x4b + 0x0D + 0x87 + new random code（2Bytes）+ event offset(2Bytes/start from 0，0xFFFF means the current response is the end of response. Note:the data contain in 0xFFFF frame is means nothing) +event data（8Bytes）+ Checksum（2Bytes）

Second 8 bytes positive response:

0x4b + 0x0D + 0x07 + new random code（2Bytes）+ event offset(2Bytes/start from 0，0xFFFF means the current response is the end of response. Note:the data contain in 0xFFFF frame is means nothing) +event data（8Bytes）+ Checksum（2Bytes）

Event read done confirm frame

This frame is used to tell the lock, which events is being read done.

Request：

0x3A + 0x07+ 0x08 + new random code（2Bytes）+ first confirm offset（2Bytes/start from zero）+ count（2Bytes）+ Checksum（2Bytes）

Response：

0x4b + 0x04 + 0x08 + new random code（2Bytes）+ error code（1Byte）+ Checksum（2Bytes）